OBLIQUE MERCATOR

10xxxxxx.1

# LEGAL RESTRICTIONS:

This Reusable Software Component (RSC) contains data with Unlimited Government Rights.

# DESCRIPTION:

OBLIQUE MERCATOR is a C language code component that provides conversions between Geodetic coordinates (latitude and longitude) and Oblique Mercator projection coordinates (easting and northing).

# CERTIFICATION LEVEL:

This RSC has been certified at level 4.  A level 4 component satisfies the criteria for reliability, testing, and documentation for the Army Reuse Center (ARC).  The component comes with test materials and a Reuse Manual that aids in integrating the component into a software system.

# LEVEL OF TESTING/ACCEPTANCE:

Unit and integration testing have been performed for the functions contained in this component.

# PURPOSE/INTENDED USE:

The purpose of OBLIQUE MERCATOR is to provide a reusable component that supports the following coordinate conversions:

- Geodetic coordinates (latitude and longitude in radians) to Oblique Mercator projection coordinates (easting and northing in meters), and

- Oblique Mercator projection coordinates (easting and northing in meters) to Geodetic coordinates (latitude and longitude in radians).

A particular ellipsoid is specified in terms of the following parameters:

- Semi-Major Axis (a):	Radius (in meters) at the equator, and

- Flattening (f):	Ratio of the difference between the semi-major axis and polar radius of the Earth to its semi-major axis.

A particular variation of the Oblique Mercator projection is specified in terms of the following parameters:

- Origin Latitude – Latitude (in radians) at which the scale factor of the projection is 1.0,

- Latitude 1 – Latitude (in radians) of the first point lying on the central line, at which the scale factor of the projection is 1.0,

- Longitude 1 – Longitude (in radians) of the first point lying on the central line,

- Latitude 2 – Latitude (in radians) of the second point lying on the central line, at which the scale factor of the projection is 1.0,

- Longitude 2 – Longitude (in radians) of the second point lying on the central line,

- False Easting – A coordinate value (in meters) assigned to the central meridian of the projection to avoid the inconvenience of using negative coordinates,

- False Northing – A coordinate value (in meters) assigned to the origin latitude of the projection to avoid the inconvenience of using negative coordinates, and

- Scale Factor – a multiplier for reducing a distance in projected coordinates to the actual distance along the central meridian.

# HARDWARE/ENVIRONMENT CONSTRAINTS:

There are no hardware or environment constraints.  There are no limitations.

# FUNCTIONS:

Set_Oblique_Mercator_Parameters – This function sets the ellipsoid parameters and Oblique Mercator projection parameters for the particular variation of the Oblique Mercator projection that is to be used in subsequent coordinate conversion operations.

Get_Oblique_Mercator_Parameters – This function returns the current values of the ellipsoid parameters and Oblique Mercator projection parameters.

Convert_Geodetic_To_Oblique_Mercator – This function converts the specified geodetic coordinates (latitude and longitude in radians) to Oblique Mercator projection coordinates (easting and northing in meters) using the current ellipsoid parameters and Oblique Mercator projection parameters.

Convert_Oblique_Mercator_To_Geodetic – This function converts the specified Oblique Mercator projection coordinates (easting and northing in meters) to geodetic coordinates (latitude and longitude in radians) using the current ellipsoid parameters and Oblique Mercator projection parameters.

# EXAMPLE ApplicationS:

The following example illustrates how Oblique Mercator can be used to convert Geodetic coordinates to Oblique Mercator projection coordinates and back again:

## Function Call:

status = Set_Oblique_Mercator_Parameters (a, f, Origin_Latitude, Latitude_1, Longitude_1, Latitude_2, Longitude_2, False_Easting, False_Northing, Scale_Factor)

## Inputs:

- a	6378137.0  (WGS84)

- f	1 / 298.257223563  (WGS84)

- Origin_Latitude	45.0

- Latitude_1	40.0

- Longitude_1	0.0

- Latitude_2	50.0

- Longitude_1	0.0

- False_Easting	0.0

- False_Northing	0.0

- Scale_Factor               1.0

## Outputs:

- None

## Function Call:

status = Convert_Geodetic_To_Oblique_Mercator (Latitude, Longitude, Easting, Northing)

## Inputs:

- Latitude:	-35.0

- Longitude:	75.0

## Outputs:

- Easting:	6906309.86

- Northing:	-7746842.76

## Function Call:

status = Convert_Oblique_Mercator_To_Geodetic (Easting, Northing, Latitude, Longitude)

## Inputs:

- Easting:	6906309.86

- Northing:	-7746842.76

## Outputs:

- Latitude:	-35.0

- Longitude:	75.0