/*!

\page create_cylinder_page Cylinder

\n To create a \b Cylinder in the <b>Main Menu</b> select <b>New

Entity - > Primitives - > Cylinder</b>

\n There are 2 algorithms for creation of a \b Cylinder.

\n The \b Result of each operation will be a GEOM_Object (SOLID).

\n Firstly, you can define a \b Cylinder by the <b>Base Point</b> (the central

point of the cylinder base), the \b Vector (the axis of the cylinder),

and its dimensions: the Radius and the Height.

\n <b>TUI Command:</b> <em>geompy.MakeCylinder(Point, Axis, Radius, Height)</em>

\n <b>Arguments:</b>  Name + 1 vertex + 1 vector + 2 values

(Dimensions: radius and height).

\image html cylinder1.png

\n Secondly, you can define a \b Cylinder by the given radius and the

height at  the origin of coordinate system. The axis of the cylinder

will be collinear to the OZ axis of the coordinate system.

\n <b>TUI Command:</b> <em>geompy.MakeCylinderRH(Radius, Height)</em>

\n <b>Arguments:</b> Name + 2 values (Dimensions at origin: radius and

height).

\image html cylinder2.png

<b>Example:</b>

\image html cylinders.png "Cylinders"

Our <b>TUI Scripts</b> provide you with useful examples of creation of

\ref tui_creation_cylinder "Primitives".

*/