/*!

\page glue_faces_operation_page Glue Faces

\n To <b>Glue Faces</b> in the <b>Main Menu</b> select <b>Repair - > Glue Faces</b>.

\n This operation glues faces that are coincident with respect to the

given tolerance value.

\n The \b Result will be a \b GEOM_Object.

\n <b>TUI Command:</b>

<em>geompy.MakeGlueFaces(theShape,theTolerance)</em>,

where \em theShape is a compound of solids to be glued, \em

theTolerance is a maximum distance between two faces, which can be

considered as coincident.

\n <b>Arguments:</b> Name + Compound of solids + Tolerance value.

\n <b>Advanced option:</b>

\ref restore_presentation_parameters_page "Set presentation parameters and subshapes from arguments".

\image html glue1.png

\n It is also possible to manually select the faces that will be

glued - select the shape, specify the tolerance and press \b Detect button.

\image html glue2.png

\n \b Geometry module detects the faces where gluing can be performed and

displays a notification.

\image html glue3.png

\n The faces that can be glued are colored in red.  It is possible to

select the faces for gluing in the 3D viewer. The selected faces will

be marked in white.

\n <b>Example:</b>

\image html glue_faces1.png

<center><em>Object that needs gluing</em></center>

\image html glue_faces2.png

<center><em>Glued object</em></center>

\image html glue_faces3.png

<center><em>Manual selection of faces for gluing</em></center>

Our <b>TUI Scripts</b> provide you with useful examples of the use of

\ref tui_glue_faces "Repairing Operations".

*/