/*!

\page scalar_map_page Scalar Map presentation

\n \image html scalar_map_on_points.png "Scalar map on Point Mesh"

\n <b>Scalar Map</b> is the basic type of visualization of calculation

data in \b Post-Pro module. This presentation consists of the

following:

<ul>

<li>creating a 3D presentation on the basis of a field:</li>

<ul>

<li>on the whole mesh (default)</li>

<li>on group(s) of the mesh</li>

</ul>

<li>coloring the cells of this presentation, according the scalar or

vector values applied to this cells;</li>

<li>creating a scalar bar representing the correspondence  between the

scalar values of the field and the used colors.</li>

</ul>

All other types of field presentations are based on the <b>Scalar Map</b>.

<em>To create a Scalar Map presentation:</em>

\par

&ndash; Right-click on one of the time stamps of the field in the Object

browser and from the pop-up menu choose <b>Scalar Map</b>, or

<br>

&ndash; Click on one of the time stamps of the field in the Object browser

and select from the main menu <b>Visualization > Scalar Map</b>, or

click <em>"Scalar Map"</em> icon in the <b>Visualization

Toolbar</b>

\image html scalarmapicon.jpg

<center><em>"Scalar Map" icon</em></center>

\par

The following dialog box will appear:

\image html scalarbarproperties.png

\note This dialog box is the same for all types of field presentations

and it will be present as an additional tab in all dialog boxes

allowing to create the other types of presentations. It allows to set

the parameters of <b>Scalar range</b>, as well as the parameters of

the <b>Scalar Bar</b>. You can also set these parameters as

preferences which will be used in all SALOME sessions: in the main

menu select <b>File > Preferences > Post Pro > Scalar Bar</b>.

\par

<br>

<ul>

<li><b>Build presentation on groups:</b> You can choose the groups on

which <b>Scalar Map</b> can be built from the list to the left and add

them in the list to the right.</li>

<li><b>Scalar range parameters:</b></li>

<ul>

<li><b>Scalar Mode</b> field: This field allows to choose the mode of

transformation of vector values into scalar values for creation of the

presentation. It can be module of the vector or one of three

components of the vector. (Note: This choose box is relevant only for

fields containing vector values, in other cases it will be

inactive).</li>

<li><b>Logarithmic scaling</b> check box: You can apply logarithmic

scaling to the color scale of the presentation.</li>

<li><b>Gauss Metric</b> field: This field allows to choose what kind

of value will be extracted from gauss points data. (Note: This field

is shown only for two presentation types - <b>Scalar Map</b>

and <b>Deformed Shape and Scalar Map</b>, and it is active only when

the result is based on multiple gauss points).</li>

<li><b>Use field range:</b> if this check box is selected, all cells

of the field presentation will be colored according the values applied

to these cells.</li>

<li><b>Use imposed range:</b> here you can define your own range of

scalar values of the field, on the basis of which this presentation

will be constructed. The cells of the field presentation with applied

scalar or vector values belonging to this imposed range will be

colored according to these values. The other cells of the field

presentation (with applied scalar or vector values out of this imposed

range) will be displayed using one color. For the cells with applied

scalar or vector values, which are less than the minimal value of the

imposed range, the color will be the same as for the cells with

applied minimal values. For the cells with applied scalar or vector

values, which are larger than the maximal value of the imposed range

the color will be the same as for the cells with applied minimal

values.</li>

</ul>

<li><b>Colors and Labels:</b></li>

<ul>

<li><b>Nb. of colors</b> field: In this field you can enter the number

of colors, which will be used for coloring of cells containing scalar

values.</li>

<li><b>Nb. of labels</b> field: In this field you can enter the number

of labels, indicating graduation of the scalar bar.</li>

</ul>

<li><b>Orientation:</b> You can select \b Vertical or \b Horizontal

orientation of the scalar bar.</li>

<li><b>Origin:</b> you can enter X & Y coordinates of the origin of

the scalar bar.</li>

<li><b>Dimensions:</b> you can enter \b Width and \b Height of the

scalar bar specified in % of the size of view.</li>

<li><b>Text properties</b> button allows you to define from the

following dialog box:

\image html textproperty.png

<ul>

<li>\b Title of the field, on the basis of which this presentation is

created,</li>

<li>Font properties of the \b Title, displayed in the Viewer,</li>

<li>Font properties of the \b Labels, displayed in the Viewer.</li>

</ul>

</li>

<li><b>Bar properties</b> button allows you to define from the

following dialog box:

\image html barproperty.png

<ul>

<li>Relative \b size of \b Title (in percents of the width of scalar bar widget),</li>

<li>Relative \b size of \b Labels (in percents of the width of scalar bar widget),</li>

<li>Relative \b width and \b height of \b Bar (in percents of the width and height of scalar bar widget),</li>

<li>Visibility of \b Units in \b Title,</li>

<li>Precision of \b Labels which specifies the number of characters to be printed, the number of decimal places, or the number of significant digits</li>

</ul>

</li>

<li> <b> Preview </b> checkbox allows to see a preview of the

presentation while editing <b>Scalar Bar</b> properties. </li>

<li> <b> Show distribution </b> checkbox introduces additional curved

scalar bar representing the distribution curve according to the number

of mesh elements of the corresponding color. This bar is always placed

along the scalar bar.</li>

\image html distributionbar.png "Distribution Bar"

<li> <b>Save as default </b> checkbox saves the modified

<b>Scalar Bar</b> parameters as default for all other

presentations.</li>

</ul>

\note The following option is relevant only for Gauss Points presentations.

\n Gauss Points Presentations presume that two different scalar bars

are displayed simultaneously:

\n The Global scale corresponds to the minimum and maximum values

founded for the variable at all time stamps (located on the left

side). When active, the second is displayed in grey and disabled. The

Local scale defines the minimum and maximum values at the current time

stamp for the variable (located on the right side). When the Local

scale is active, the Global scale shows its limits with two black

lines.

\image html activebar2.png "Local and Global bars"

Local and Global bars can be switched in the <b>Scalar bar</b> tab for

Gauss Points presentation.

\image html activebar1.png

<ul>

<li>\b Local: when selected, the \b Local bar will be active and

colors used for rendering of the results will be taken from this

bar. If displayed, the \b Global bar will be displayed in grey and 2

markers will define the limits of the local min/max range.</li>

<li>\b Global: when selected, the \b Global bar will be active and

colors used for rendering of the results will be taken from this bar

(i.e.: not all colors may be used to display results at a given time

stamp). \b Local bar will be displayed in grey  and inactive.</li>

<li>\b Displayed: when checked, the \b Global bar is displayed, else

it is not seen. This control is systematically checked and disabled

when the \b Global bar is defined as active.</li>

</ul>

After you have finished with setting the parameters, click \b

OK. The presentation will be immediately displayed in the viewer:

\image html scalar_map_on_cells.png "Scalar map on Cell Mesh"

*/